function [duty,iterations,Wolf_Pos] = IRBMO_IPO(vpv,ipv)

persistent dc fitness alpha alpha_pos iteration i counter duty_old vpv_old Pold Max_iter;

if(isempty(iteration))

iteration=0;

counter=1;

end

if(isempty(duty_old))

duty_old=0;

vpv_old=0;

Pold=0;

end

if(isempty(alpha))

alpha=0;

end

if(isempty(alpha))

alpha=0;

end

if (isempty(alpha_pos))

alpha_pos=0.5;

end

if(isempty(dc))

dc=linspace(0,0.5,10);

end

if (isempty(i))

i=1;

end

iterations=iteration;

Wolf_Pos = dc;

if(counter>=1 && counter<30)

duty=dc(i);

counter=counter+1;

if iteration==5

Pold=vpv*ipv;

end

return;

end

Max_iter=5;

counter=1;

if (iteration<Max_iter)

iterations=iteration;

if i<10

fitness=vpv*ipv;

if fitness>alpha

alpha=fitness;

alpha_pos=dc(i);

end

i=i+1;

if i<11

duty=dc(i);

else

duty=dc(10);

end

return;

end

iteration = iteration+1;

i = 1;

fitness=vpv*ipv;

dc1 = pos_up(iteration,alpha_pos,dc,Max_iter,fitness);

dc=dc1;

else

counter=0;

deltaD=0.0001;

duty_max=0.95;

duty_min=0.05;

P=vpv*ipv;

dP=P-Pold;

if abs(dP)>0.05*Pold

iteration=0;

counter=1;

alpha=0;

alpha_pos=0;

dc=linspace(0,0.5,10);

i=1;

end

dV=vpv-vpv_old;

maxStep = 0.0003;

minStep = 0.0001;

currentError = abs(dP);

beta = 2;

deltaD = minStep + (maxStep - minStep) * exp(-beta * currentError);

deltaD = min(max(deltaD, minStep), maxStep);

if dP>0

if dV>0

duty=duty_old-deltaD;

else

duty=duty_old+deltaD;

end

else

if dV>0

duty=duty_old+deltaD;

else

duty=duty_old-deltaD;

end

end

if duty>duty_max

duty=duty_max;

end

if duty<duty_min

duty=duty_min;

end

Pold=P;

vpv_old=vpv;

duty_old=duty;

return;

end

duty = alpha_pos;

duty_old = alpha_pos;

end

function [fit, X, fit_old, X_old] = Food_storage(fit, X, fit_old, X_old)

Inx = (fit_old < fit);

Indx = repmat(Inx, 1, size(X, 2));

X = Indx .* X_old + ~Indx .* X;

fit = Inx .* fit_old + ~Inx .* fit;

fit_old = fit;

X_old = X;

end

function o=Levy(d)

beta=1.5;

sigma=(gamma(1+beta)*sin(pi*beta/2)/(gamma((1+beta)/2)*beta*2^((beta-1)/2)))^(1/beta);

u=randn(1,d)*sigma;v=randn(1,d);step=u./abs(v).^(1/beta);

o=0.01*step;

end

function D1=pos_up(it,alpha_pos,D_cur,Max_iter,fitness)

Epsilon = 0.5;

dim=1;

X=D_cur;

Xfood=alpha_pos;

N=10;

D1 = zeros(1,10);

ub = 1;

lb = 0;

fitness_old = fitness;

X_old = D_cur;

for i=1:N

p = randi([2, 4]);

selected_index_p = randperm(N, p);

Xp = X(:, selected_index_p);

Xpmean = mean(Xp)

q = randi([5, N]);

selected_index_q = randperm(N, q);

Xq = X(:, selected_index_q);

Xqmean = mean(Xq);

A = randperm(N);

R1 = A(1);

R2 = A(2);

if rand < Epsilon

X(1, i) = Xfood + (Xpmean - Xqmean) .* rand + (X(1, R1) - X(1, R2)) .* rand;

else

X(1, i) = Xfood + (X(1, R1) - X(1, R2)) .* Levy(dim);

end

Flag4ub=X(1,i)>ub;

Flag4lb=X(1,i)<lb;

X(1,i)=(X(1,i).*(~(Flag4ub+Flag4lb)))+ub.*Flag4ub+lb.*Flag4lb;

end

[fitness, X, fitness_old, X_old] = Food_storage(fitness, X, fitness_old, X_old);

CF = (1 - it / Max_iter)^(2 * it / Max_iter);

for i=1:N

p = randi([2, 4]);

selected_index_p = randperm(N, p);

Xp = X(:, selected_index_p);

Xpmean = mean(Xp);

q = randi([5, N]);

selected_index_q = randperm(N, q);

Xq = X(:, selected_index_q);

Xqmean = mean(Xq);

if rand() < Epsilon

X(1, i) = Xfood + CF * (Xpmean - X(1, i)) .* randn(1, dim);

else

X(1, i) = Xfood + CF * (Xqmean - X(1, i)) .* randn(1, dim);

end

D1(i)=X(1,i);

if D1(i)>1

D1(i)=0.4;

end

if D1(i)<0

D1(i)=0.3;

end

[fitness, X, fitness_old, X_old] = Food_storage(fitness, X, fitness_old, X_old);

end

end